Digital Twin City

Project Research & Development Log

Mayanglambam Premananda Singh  ·  v0.3 — March 2026  ·  Living Document

# 1. Vision & Inspiration

The goal of this project is to build a digital twin of a real urban environment — a navigable, data-faithful 3D representation of a city or district that can serve as a platform for urban planning, infrastructure management, real estate visualisation, disaster simulation, and civic decision-making.

1.1  Reference: 51World (Shanghai)

The primary visual and technical inspiration is the Shanghai digital twin built by 51World, a Beijing-based spatial intelligence company. Their implementation uses Unreal Engine and proprietary algorithms to convert 2D geographic data into a photorealistic, navigable 3D cityscape — used for urban policing, city management, and infrastructure planning. The Shanghai model represents the upper end of what this project aims to reach at full scale.

51World's technology demonstrates that photorealistic digital twins are achievable from geographic data — not just from expensive LiDAR hardware. The rendering pipeline is the valuable IP, not the raw data collection.

1.2  Reference: Shanghai Government Twin (Surveying & Mapping Institute)

A separate government-grade implementation by the Shanghai Surveying and Mapping Institute achieves 3cm positional accuracy using airborne laser scans and street-level LiDAR. This is the accuracy benchmark for the advanced version of this project. It is not the target for the current research phase — it represents where the project can go with the right funding and hardware.

# 2. Methodology Evolution

The project has moved through two distinct phases, with a third planned post-funding. This evolution reflects real research progress rather than a planned roadmap — each stage was driven by what was available, what was learned, and what actually worked.

2.1  Phase 1 — Web MVP (Procedural Generation)

The initial approach was a zero-budget web MVP using entirely open-source, browser-based tools:

- Mapgen4  — Open-source JS/TS procedural terrain generation — rivers, biomes, elevation algorithms

- MetroMap.io  — Procedural city layout around geographic constraints — rivers, coastlines, islands

- CGA Shape Grammars  — Academic framework for rule-based 3D building generation from 2D footprints

- Google AlphaEarth (Earth Engine)  — AI satellite embeddings for land classification and real-world geographic grounding

- Three.js  — Low-poly 3D cityscape rendering in the browser

This phase proved the concept and defined the visual target. The procedural approach works for investor demos and proof-of-concept, but produces stylised results that are not geometrically faithful to real locations.

2.2  Phase 2 — Blender + GIS (Current)

The current research phase replaces procedural generation with real geographic data processed through a professional 3D pipeline:

- Blender 4.x + BlenderGIS addon  — Direct import of OSM vector data, elevation rasters, and satellite imagery into a 3D scene

- OpenStreetMap  — Real building footprints, road networks, land use — geometrically accurate to the actual city layout

- SRTM / Copernicus DEM  — 30m resolution elevation data for accurate terrain and topography

- QGIS  — GIS preprocessing — coordinate system management, shapefile editing, data cleaning before Blender import

- Google Earth Engine  — Continued exploration of AlphaEarth embeddings for AI-driven land classification and change detection

This phase is producing geometrically real 3D city geometry — building footprints are actual buildings from OSM data, terrain follows real elevation. The visual fidelity is low-poly but the data accuracy is genuine. This is the current active research stage.

2.3  Phase 3 — Funded Pipeline (Planned)

Post-funding, the pipeline upgrades to higher-fidelity data capture and a production rendering engine:

- Esri CityEngine  — Shape grammar-based mass building generation at city scale from footprint + height data

- Drone photogrammetry  — Close-range photorealistic 3D reconstruction of specific buildings or districts

- LiDAR scanning  — High-precision point cloud capture for sub-metre accuracy where budget allows

- Unreal Engine 5  — Photorealistic rendering, real-time simulation, and potential AR/VR output

The specific data collection method (drone vs LiDAR vs commercial satellite) will be selected based on purpose and available budget at that stage — the architecture is designed to accept any of these inputs.

# 3. Current Tool Stack & Data Sources

| Tool / Source | Role | Status / Notes |
| --- | --- | --- |
| Blender 5.x | Primary 3D authoring + visualisation | Active — main working environment |
| BlenderGIS addon | Import GIS data directly into Blender scene | Active — loading OSM + elevation data |
| QGIS | GIS data preprocessing, CRS management, shapefile editing | Learning — used for data prep |
| OpenStreetMap | Building footprints, roads, land use polygons | Active data source |
| SRTM / Copernicus DEM | 30m resolution elevation / terrain data | Integrated via BlenderGIS |
| Google Earth Engine | AlphaEarth AI embeddings, land classification, change detection | Explored — access confirmed |
| Mapgen4 | Procedural terrain and river generation reference | Reference / prior MVP stage |
| MetroMap.io | Web-based procedural city layout reference | Reference / prior MVP stage |
| CityEngine (ArcGIS) | Shape grammar-based mass 3D building generation | Post-funding target |
| Unreal Engine 5 | Photorealistic rendering and real-time simulation | Advanced version target |

# 4. Feature Roadmap by Phase

| Feature | MVP — Blender/GIS | Funded v1 | Advanced Vision |
| --- | --- | --- | --- |
| Data Source | OpenStreetMap + SRTM elevation + Google Earth Engine AlphaEarth embeddings | Commercial satellite imagery, government shapefiles | Aerial LiDAR surveys, drone photogrammetry, IoT sensor feeds |
| 3D Generation | BlenderGIS → OSM building footprints → procedural extrusion | CityEngine shape grammars + semi-automated texturing | Unreal Engine 5 photorealistic rendering pipeline |
| Accuracy | Stylized, data-faithful (building footprints real, heights approximate) | Sub-metre positional, realistic materials | 3cm LiDAR accuracy (Shanghai-grade) |
| Simulation | Static navigation and exploration | Basic traffic + shadow analysis | Real-time data streams: traffic, weather, emergency response |
| Infrastructure | Single machine, Blender + QGIS + free APIs | Cloud-hosted tile server, PostGIS database | Distributed rendering cluster, real-time ingestion pipeline |
| Cost | Zero — fully open source | ~₹5–20L setup | Enterprise scale |

# 5. Open Research Questions

The following questions are actively being investigated. Answers will shape the next technical decisions:

Data & Coverage

- Which city or district to use as the primary test case — needs sufficient OSM coverage and a compelling use case

- How to best handle buildings with missing height data in OSM (majority of Indian cities)

- Whether ISRO Bhuvan or Survey of India data can supplement OSM for Indian urban areas

- How useful AlphaEarth embeddings are for height estimation vs simple heuristics

Pipeline & Workflow

- Optimal coordinate reference system (CRS) workflow between QGIS and Blender

- How to manage scene complexity as city area scales — LOD strategies in Blender

- Whether to build a Python scripting layer in Blender for automated data import and geometry generation

Use Case Focus

- Which vertical to target for the funded version — urban planning, real estate, telecom (5G tower placement), or disaster response

- Whether the MVP should be interactive (web viewer) or a rendered flythrough video for the initial investor pitch

# 6. Target Customer Segments

The MVP and early funded versions are positioned for customers underserved by expensive enterprise solutions:

- Local real estate developers  — Visualise new projects in the context of a real neighbourhood before construction

- Small and mid-size city planning departments  — Urban simulation and planning tools without enterprise software budgets

- Telecom companies  — 5G tower placement planning using accurate 3D line-of-sight analysis

- Disaster response agencies  — Evacuation route simulation and infrastructure vulnerability mapping

- Smart city initiatives  — Data visualisation layer for IoT sensor networks and city dashboards

Target segment is still being validated. The funded version will focus on one vertical based on which customer shows the strongest demand and ability to pay.

# 7. What the Current Work Proves

Even at this early research stage, the Blender + GIS work demonstrates several things that matter to future investors and collaborators:

- Technical credibility  — Ability to acquire, process, and render real geographic data into a 3D environment — not just a concept

- Resourcefulness  — Achieved meaningful output with zero budget using open-source tooling and public data

- Scalable architecture thinking  — The pipeline is designed to swap in higher-quality data sources and rendering engines as funding arrives — not a dead end

- Domain depth  — Active learning of GIS systems, coordinate reference systems, and spatial data processing — not just a visual demo

- Clear path to scale  — Phase 3 tools (CityEngine, LiDAR, UE5) are well-defined and the upgrade path is credible

# 8. Changelog

v0.3 — March 2026

- Blender + BlenderGIS pipeline working with real OSM data

- 3D city geometry generated from real building footprints

- QGIS integrated for data preprocessing

- AlphaEarth / Google Earth Engine access confirmed and explored

- Project renamed to Seva Mitra

v0.2 — February 2026

- Procedural web MVP scoped and documented

- Mapgen4, MetroMap.io, CGA Shape Grammars researched

- Three.js rendering pipeline defined

- MVP strategy document written

v0.1 — Initial

- Project inspired by 51World Shanghai digital twin

- 51World and Shanghai Surveying & Mapping Institute references documented

- Initial scope and investor pitch rationale defined

This is a living document — updated as research progresses. Last updated March 2026.